Original:   1979

Revised: 05jan82

hv1.1:   20nov82   <

05jan83

01mar84

hv2.0:   26jan88   &

05apr88

16feb91

22oct92   |

BBF-Format ESG Time Series Data Files

Table of Contents

=================

1.0  Introduction

2.0  File Structure

3.0  Time Series Samples

4.0  Direct-Access I/O

5.0  Some Programming Techniques

6.0  Sample Program

7.0  File Naming Conventions

7.1  DRD (=Digitally Recorded Data) File Names

7.2  AGRAM Data File Names

8.0  Header Blocks

8.1  First Integer Header Block

8.2  First Real Header Block

9.0  BBFILE and TSPLOT: programs that display the

Contents of Blocked Binary Data Files

1. Introduction

===============

The content of most of the time series data files collected and

maintained by the USGS Branch of Enginering Geology and Seismology are

all arranged in a similar, somewhat standard format.  The files are

sometimes referred to as "blocked binary data files" or some

abbreviation thereof, like "bbf", because the files are arranged in

fixed-sized blocks of binary data.  The files are also sometimes

referred to as "DR100" files because it was the data recorded with

Sprengnether DR100 digital recorders that was the first to be stored

in the blocked binary form that later came to be used for almost all

the digital seismograms processed by the Branch.

Seismograms from a variety of recorders are currently maintained

in the blocked binary data files.  The file format, especially the

contents of the header blocks, has evolved to accomodate various

sources of data and various methods for processing the data.  The

waveforms come from two principal sources: permanently-installed, and

portable recording instruments.  The permanently-installed strong-

motion recorders are usually analog accelerographs producing records

that must be digitized before they can be processed with a computer.

The portable instruments record acceleration and/or velocity in digital

form.  The digitized analog data and the digtal-recorded data are

processed rather differently in the initial stages of their routine

computer processing and the content of the resulting time series data

files reflect these differences to some extent.  The processed analog

data files are called "AGRAM" data files (after the name of the

processing software that produces them) to distinguish them from the

***played-back, tidied*** "DRD", or digitally recorded data files.

April Converse (ext. 5666) and Peter Mork (ext. 5671) are

responsible for most of the AGRAM data files.  Chuck Mueller (ext.

5646) co-ordinates the content and eventual archive storage of most

the DRD files.  April occasionally updates this text to describe the

current content of the AGRAM files and attempts to indicate where and

how the AGRAM files differ from the DRD files.  Chuck Mueller, however,

has a much better understanding of what is in the various digitally

recorded data files.  He maintains a hand-out that describes the

contents of the header block locations that he may update independently

of this text, so if you are dealing with digitally recorded data, it

would be a good idea to speak with Chuck.  John Watson, Gary Maxwell,

Gary Glassmoyer, and Linda Seekins are also familiar with the DRD files.

Although he has not had time to do so yet, Chuck will eventually

read over this text to make sure that April's descriptions of DRD

related topics is accurate.  In the meantime, many of the statements that

April is uncertain about are flagged with three asterisks (***) to

let readers know that Chuck may have more information about the topic

or may disagree with the way it is presented here.

2.  File Structure

==================

Each file is a sequence of 512-byte blocks.  Each block contains

256 two-byte binary integer values, 128 four-byte binary real (=

"floating-point") values, or, very rarely, 512 ASCII characters.  The

first several blocks in each file are "header" blocks that contain

auxiliary information about the time series samples contained in the

remaining "data" blocks.  Each file has a minimum of two header

blocks, the first of which contains 256 integer values and the second

of which contains 128 real values.  The time series samples stored in

the data blocks may be in either integer or real format.  The AGRAM

files contain real time series samples and the DRD files usually

contain integer time series samples.

A single waveform is stored in each file, with the exception of the

AGRAM/BUTTER output files.  Each BUTTER output file contains data for

all the traces on a digitized record, including the reference traces

and the time trace.

3. Time Series Samples

======================

The data blocks in the DRD files, and in most of the AGRAM files

as well, contain single-valued samples evenly spaced in time.  The

sample values (real numbers) in most the AGRAM files represent

cm/sec/sec, cm/sec, or cm, depending on whether it is an acceleration,

velocity or displacement data file.  The samples (integers) in the DRD

files represent instrument counts that can be converted to other units

using instrument characteristic information that is stored in the

header blocks or in separate "DBS" files.  The factor, F, used to

convert instrument counts to cm/sec/sec or cm/sec (depending on

whether it is acceleration or velocity data) is

F = 1.0/( C * V * G )

where:    C = coil constant from the .DBS file, from the file header

at RHEAD(51), or default value of .5 for acceleration data

or .0068 for velocity data

V = counts per volt from the .DBS file, from the file header

at RHEAD(46), or default value = 204.8

G = gain from the .DBS file, or

= 10.0**(gain/20.) when the gain value comes from the file

header at RHEAD (52), or

= 128.0, the default value used when no gain is given in

a .DBS file or the data file header.

*** OOps, do we need the pre-amp gain, RHEAD(53) in here

too? ***

Note that when the gain value is found in a .DBS file it is a factor,

when it is found in the header block of the data file it is in

decibels.  Although it is not quite ready for general use, there is a

subroutine, DIGDAT, on the AGRAMLIB library that can be used to check

for the existence of a .DBS file; retrieve the C,V, and G values from

the .DBS file, the data file header blocks, or the default list; then

calculate the instrument-counts-to-cm/sec/sec conversion factor.  If

you'd like a copy of a preliminary version of the subroutine, talk to

April.

*** when and where are there .DBS files? ***

Unlike the equally sampled data in most the blocked binary data

files, the files produced by the first two programs, BUTTER and SCALE,

in the AGRAM process contain unequally spaced samples and consequently

contain two values (x and y) for each sample.  In BUTTER output files,

x and y are in digitization units (=microns if the digitization was

done at IOM/TOWILL).  In SCALE output files, x is in seconds, and y is

scaled, uncorrected instrument response in cm/sec/sec.

4.  Direct-access I/O

=====================

The blocked binary files need not be read sequentially; the

calling program can access any block in any sequence.  In FORTRAN,

the file access can be done through statements like:

lundat = <logical unit number to use with the file>

filnam = <file name in a character variable>

nih    = <number of integer headers in the file>

nrh    = <number of real    headers in the file>

nth    = <number of text    headers in the file>

array  = <an array into which the data will be read>

OPEN (UNIT=lundat,FILE=filnam,

X      STATUS='OLD', ACCESS='DIRECT', RECL=128,  READONLY)

READ (lundat,REC= 7+nih+nrh+nth) (array(J), J=1  ,128)

READ (lundat,REC= 3+nih+nrh+nth) (array(J), J=129,256)

CLOSE (UNIT=lundat)

In this example, the code reads the contents of the seventh data block

followed by the contents of the third data block (there's no reason why

anyone would really want to do that!) into an array named ARRAY.

Note the RECL keyword in the open statement.  RECL indicates the length

of each  block (or "record") in the file in "longwords", a longword

being 4 bytes.  The RECL portion of the OPEN statement could have

been omitted in this example because it is an existing file that is

being opened.  The RECL phrase is required when opening a new file,

however.  Note the REC= phrases in the READ statements: they indicate

which block will be read from the file.  Note also that it is real

data (i.e., AGRAM-style) that is read from the file in this example.

Code using machine-specific I/O operations in place of the FORTRAN

I/O statements illustrated above can be more efficient.  Such code can

avoid the use of FORTRAN buffers, since the i/o is always done for one

or more complete disk sectors, and pass the data directly to/from the

disk from/to the array space in the calling program.  Joanne Vinton

once wrote some subroutines to do this via some of the subroutines

provided along with the VAX/VMS operating system.  For more

information, see her instructions on the VAX/750 at TECH:BLOCKIO.MEM.

5.  Some Programming Techniques

===============================

The first header block, like any block containing integers,

contains 256 two-byte integer values.  Since integers are 4 bytes

long by default in the VAX FORTRAN, some extra attention must be

given to code that reads or writes the half-sized integers in the

blocked binary data files.  The most straight-forward approach is

to declare the array in which the integer header values will be

kept as an INTEGER*2 array, as in:

INTEGER*2 IHEAD(256)

ÿ        ...

READ (lundat,REC= 1) (IHEAD(J), J=1,256)

Unfortunately, code that is meant to be portable should not include

INTEGER*2 declarations, so another method is used in the AGRAM

programs.  The information is read into an array of 4-byte (32-bit)

integers and subroutine EXPAND (on SMASHLIB and AGRAMLIB) is called

immediately afterward to expand the 2-byte integers to 4-byte

integers. Example:

INTEGER IHEAD(256)

ÿ       ...

READ (lundat,REC= 1) (IHEAD(J), J=1,128) !read 128*4 bytes

CALL EXPAND (IHEAD,IHEAD) !now we have 256 4-byte integers

Another subroutine, SQUISH, is called immediately before writing

a block of integers to a blocked binary data file.  SQUISH compacts

the integers from 4 bytes back down to 2 bytes.  SQUISH merely

removes the higher order two-bytes in each integer, but if those

two bytes are not zero, SQUISH will not proceed, under the assumption

that by some coding error the caller is attempting to write a larger

integer than can be stored in 2 bytes onto a blocked binary data file.

This can lead to problems if the few integer header cells that contain

Hollerith data (e.g., IHEAD(210)) are loaded in an obvious way.

Neither

IHEAD(210) = 2HAB

or    IHEAD(210) = 4H  AB

will work well, for they both leave non-zero bits in the first (highest

numeric significance) half of the IHEAD(210) cell.  So another

SMASHLIB/AGRAMLIB subroutine, PUT2C, is used to insert two hollerith

characters into the second (lowest numeric significance) 2 bytes of a

4-byte integer, leaving the first 2 bytes as zero so subroutine SQUISH

can handle that header block cell just like any other integer.

Subroutine GET2C is the retrieving counterpart to PUT2C.

Illustration:

On the VAX, i =   2HAB     leaves i = 20204241 (hex),

i = 4H  AB     leaves i = 42412020 (hex),

call put2c('AB',i) leaves i = 00004241 (hex),

where hex 20 represents the blank character.

6.  Sample Program

==================

There is a little sample program named BBDATA that illustrates

how to read and write blocked binary data files.  BBDATA does nothing

but read a blocked binary data file, plot the data, alter the data

slightly by adding a constant to each sample, then write another

blocked binary data file.  It uses subroutines FREAD2 and FWRIT2 to

read and write the files;  these subroutines can be called, as is, from

other programs or their code can be used as examples.  The calling

program in BBDATA.FOR and linking command in BBDATA.LNK can be used as

a template or first draft when creating a new program that manipulates

blocked binary data files.

The program, consisting of BBDATA.FOR, BBDATA.LNK, and several

.INC include files referenced from BBDATA.FOR, is on the VAX/750 in

the directory at GSVAX1::PUB1:[AGRAM.SAMPLE.BBDATA].  The reading and

writing subroutines, FREAD2 and FWRIT2, are on the SMASHLIB library on

all 3 VAXes (they will soon be on the newer, more portable AGRAMLIB

library too).  For an example of how to link to the SMASHLIB library,

see the BBDATA.LNK file.  For general instructions about BBDATA, FREAD,

and FWRITE, see the text file at GSVAX1::PUB1:[AGRAM.DOCS]BBDATA.DOC.

7.   File Naming Conventions

============================

7.1  DRD (=Digitally recorded data) file names

==============================================

The DRD files usually have names that indicate where and when the

record occured.  They take the form:  JJJHHMMSC.STA,  where

JJJ is the  Julian day,

HH is the hour,

MM is the minute,

S is the second

(in letters A-T, each letter representing a 3 second

interval),

C is the component number

(this number is 1, 2, or 3 for acceleration;

4, 5, or 6 for velocity;

and 7, 8, or 9 for displacement.

and STA is the 3 letter station name.

Some examples: ****

7.2  AGRAM Data File Names

==========================

The routine, preliminary processing for the AGRAM data is rather

different than that for the digitally recorded data.  Unlike the

digitally recorded data for which there is usually just a single file

maintained  on the disk for each waveform, there are several files for

each waveform processed by the AGRAM programs, each of which represents

the waveform after a different stage in the processing.  Because there

are several files per waveform (and for other reasons too!), the files

processed by the AGRAM programs generally do not conform to the

digitally recorded data naming convention.  The files that result at the

end of the AGRAM process could be renamed once processing is complete

(they could be renamed, but I don't think they are, as a rule), but

during processing the names are:

anything.BUT   for BUTTER output files,

"     .R0n   for SCALE output files

(the n is replaced by the component

number: 1, 2,or 3),

"     .G0n   for HIFRIC output files,

"     .A0n   for AVD or CORAVD output files,

"     .V0n        "       "                ,

and   "     .D0n        "       "                .

The file name prefix, shown as "anything" above, is some abbreviation

that suggests the earthquake and/or station involved.  It is not

difficult to select a unique name for these analog records, as it is

for the digitally recorded records, because there are so few of them.

As a result, the names selected for the AGRAM files tend to be easier

to remember and type than the DRD file names.

8. Header Blocks

================

The first few blocks in each data file are called header blocks;

they contain auxiliary information about the time-series contained in

the remaining blocks.  There are three types of header blocks: first

there are one or more integer headers, followed by one or more real

headers, and then possibly some text headers.  The contents of the

first integer header and the first real header should conform to the

conventions listed in sections 8.1 and 8.2.  Any other header blocks

exist only for the convenience of special-purpose programs and they are

not filled according to any generally followed convention.  In fact,

additional headers are very rarely used; most files have only one

integer header, one real header, and no text headers.

The lists given in sections 8.1 and 8.2 show what may be stored

in each of the cells of the two required header blocks.  Many of the

cells are rarely used and usually just contain a number that indicates

that the value is undefined.  ("Undefined" is usually represented

by -32768 in integer blocks and by +1.7e+38 or -0.3e-38 in real blocks).

Very few of the cells in the required header blocks are filled with

anything but "undefined" in the AGRAM files.  A single character in the

first column of the lists below indicates whether and how the header cell

is used by the AGRAM programs:

*    indicates that the header value is set and used by the

AGRAM programs.  Or the value will probably be

set and used in the future.

+    indicates that the header value is set by one of the

AGRAM programs for information only, the value is not

picked up and actually used by another AGRAM program.

ÿ  .    indicates that the header value is not one that is ever

set by any of the AGRAM programs, but it is one that

some AGRAM programs may use while processing a DRD file.

(blank) indicates that the header value is not set or used by any

of the AGRAM programs.

Although the AGRAM programs do not make use of many of the header

values that occur in the DRD files,  each AGRAM program does (or

should!) copy all the header values from its input file to its output

file.

Throughout this write-up, and particularly in the lists below,

a cell in the required header blocks is refered to as IHEAD(#) or

RHEAD(#), just as though IHEAD and RHEAD were arrays in a FORTRAN

program into which the first integer header block and the first real

header block of a file had been read.

A distinction is sometimes made in the lists about which

version of the header block convention is applicable.  The versions

are noted as HV1.0 (header version 1.0), HV1.1 (header version 1.1)

and HV2.0 (header version 2.0).  There are change-bars in the right-

hand margin of the lists below to indicate information that changed

with HV1.1 (change-bar = <) and HV2.0 (change-bar = &).  There is

little difference between HV1.1 and HV1.0:

- IHEAD(41) was 'HR' or 'VR' in HV1.0, changed to vertical

orientation in degrees in HV1.1.

- IHEAD(44) was + or -1 in HV1.0, ignored in HV1.1

Many more header block cells are used in HV2 than were used in HV1

and a few of the cells have a different meaning in HV2 than they did

in HV1.  The files produced by the AGRAM programs still follow the

HV1 conventions (AGRAM programs were insensitive to the difference

between HV1.0 and HV1.1); They will eventually be changed to produce

HV2 files.

Those header cells whose contents have a DIFFERENT meaning in HV2

than they do in HV1 are:

- IHEAD( 4), which indicates whether the time series samples are

2-byte integers or 4-byte reals was "undefined" or

+1 in the HV1 convention, but are -2 or +4 in the HV2

convention.

(IHEAD(4) = IHEAD(HIORR) when using HEADERLOC.INC)

- IHEAD(40)  was 'FB' or 'VT' in HV1, but is the sensor serial

number in HV2 files.

(IHEAD(40) = IHEAD(HTTYPE))

- IHEAD(43)  was 'AC', 'VL' or 'DP' in HV1, but is a sensor type

code in HV2.  The type of data identification that

the IHEAD(43) cell conveyed in HV1 files is stored

(as an integer, not as characters!) in IHEAD(254)

in HV2 files.

(IHEAD(43) = IHEAD(HAVD); IHEAD(254)= IHEAD(HDDAVD))

- RHEAD(49)  is the natural FREQUENCY of the sensor in HV2, not

the natural PERIOD as it was in HV1.

(RHEAD(49) = RHEAD(HPER))

- RHEAD(98)  is the trace mean in HV2, that is, the mean value of

the time series contained in the same file as the

header in question.  In HV1 AGRAM files, this cell

contained the mean value of the trace as it was when

produced by the SCALE program.  This value is (or

should be!) subtracted from all the sample values by

any program that processes SCALE output files.  In

HV1 convention, RHEAD(98) did not change when the data

was processed beyond SCALE by other programs like

HIFRIC or CORAVD.

(RHEAD(98) = RHEAD(HVOFFS) for HV1 = RHEAD(HVMEAN)

for HV2)

The header cells that AGRAM programs will fill in HV2 that are

not filled in HV1 are:

- IHEAD(5)   =2 => header version 2

- IHEAD(6)   =0 => header subversion 0

- IHEAD(34-36) = "playback" program information?  What shall this

be for the AGRAM programs? ***

- IHEAD(253) =recording type code, but what will that be for

AGRAM files? ***

- IHEAD(254) =1,2,3 to indicate acc, vel, or disp

Actually, this cell, IHEAD(254), should get set by

any AGRAM program that gets modified after march '88.

This change needn't wait for the day when ALL the

AGRAM programs are changed to conform to HV2

completely.

Some changes that may be made to AGRAM HV2 file headers, but that

aren't settled yet:

- RHEAD(46)  May be left undefined in HV2 AGRAM files.  In HV1 AGRAM

files, this contains the digitizing constant in

digitization units/cm  (usually = 10000).  This is the

same cell in which the digitizing constant, as instrument

counts per volt, is stored in DRD files.

Chuck feels it could lead to confusion if we use the

same cell for both quantities.  I (April) am quite

willing to move it when I convert all the AGRAM programs

to HV2, but where to?

(RHEAD(46) = RHEAD(HDUNIT))

- RHEAD(47-48)  Contains the anti-alias filter characteristics,

corner frequency in RHEAD(47) and number of poles in

RHEAD (48).  In HV1 files, the AGRAM HIFRIC program

stored the beginning of transition frequency of its

high-cut filter in RHEAD(47) and the transition

bandwidth in (48).  Since the AGRAM parameters are

somewhat different than the digital data parameters that

are stored here, Chuck thinks that we may want them

stored in different cells.

How about moving the AGRAM/HIFRIC filter corner and

bandwidth to RHEAD(114) and RHEAD(115), near where the

AGRAM/CORAVD filter parameters are stored in HV1?

(RHEAD(47) = RHEAD(HAAFC); RHEAD(48) = RHEAD(HAAFR))

- RHEAD(104 - 107):  Some of the AGRAM/CORAVD filter info may get

moved or duplicated into RHEAD(54-57).  This was

indicated in Chuck's list.  I (April) don't understand

yet why this info can't just stay where it is, since the

digital data folks don't need to use RHEAD(104-107)

for anything else and probably have no need to have

the AGRAM/CORAVD filter info in any particular place.

Perhaps it is the AGRAM/HIFRIC filter info stored in HV1

RHEAD(47) and (48), not the AGRAM/CORAVD filter info in

(104)-(107) that Chuck would like moved to RHEAD(54)-(57)

in HV2?  If so, there's no need for the Lo-cut info that

Chuck's list shows in RHEAD(54) and (55), for HIFRIC

only applies a high-cut filter.  And the HIFRIC filter

is NOT a Butterworth filter, so we want to store a

transition bandwith rather than the number of poles

Chuck's list shows for RHEAD(57).

(HLFFC,HHFFC, etc.)

In addition, some conventions used for DRD files that were never

noticed by the AGRAM programs also changed when HV2 was established:

- IHEAD(10)  was a 2-digit year in HV1, a 4-digit year in HV2.

- IHEAD(53-58) used for HV1 (GEOS?) trigger info was moved to

the real header in HV2.

- IHEAD(101-209) used for BUGGER processing is obsolete in HV2.

- IHEAD(256) sometimes contained the number of data samples,

is obsolete in HV2.

- IHEAD(39)  sometimes contained a motion type indicator (text)

that is obsolete in HV2.

A (LT) is shown in the header location lists to indicate that

a look-up table may someday be set up to show all the values and

corresponding meanings that a given header cell may take.  None of

these look-up tables exist yet.

8.1  First Integer Header Block

===============================

The first header block (like any block containing integer values)

contains 256 two-byte integer values.

----- is the cell used by the AGRAM programs?

|

|

|  header

v location       contents

- --------       --------

* IHEAD(1)       Number of optional integer header blocks that follow

this one.

* IHEAD(2)       Number of optional text header blocks that follow

the real header blocks.

* IHEAD(3)       Integer value that represents "undefined" or "null"

(usually = -32768).

* IHEAD(4)       Data format code.

HV2.0:                                                   &

= -2 if each data block contains 256 2-byte integers   &

(VAX FORTRAN's INTEGER*2), as they are in most    &

DRD files;                                        &

= +4 if each data block contains 128 4-byte            &

floating-point values (FORTRAN's REAL), as        &

they are in the AGRAM files.                      &

HV1.-:

.                  = "undefined" (= IHEAD(3)) if the data samples are

2-byte integers,

*                  = +1 if the data samples are 4-byte reals.

* IHEAD(5)       Header version number. = "undefined" (for versions 1.0   &

and 1.1) or 2 for version 2.0.                        &

* IHEAD(6)       Header subversion number.                                &

* IHEAD(7-9):    optional header format codes.  These will usually be     &

"undefined". (LT)                                        &

*      (7)       Extra-integer-header format code,                        &

*      (8)       Text-header format code,                                 &

*      (9)       Extra-real-header format code.                           &

. IHEAD(10-16): "uncorrected" time of first data sample (TFS).

For "corrected" time of first sample, add the component

sample lag, RHEAD(6), the clock correction, RHEAD(60),

and the AGRAM timing adjustment, RHEAD(90).

.      (10)      Year of the event (HV2.0 = 4 digits, as in 1988,         &

HV1.- = 2 digits, as in 88),

.      (11)      Julian day of year,

.      (12)      Hour of day,

.      (13)      Minute of hour,

.      (14)      Second of minute,

.      (15)      Millisecond of second,

.      (16)      microsecond of second.

IHEAD(17-19):  information about the time ticks, or "glitches" that

are embedded in (some? all?) the DR100 time series.

(17)      Sample number of the first tick,

(18)      Amplitude of tick mark (counts),                         &

(19)      Number of tick marks detected.                           &

IHEAD(20)      Recorder serial number

IHEAD(21-22):  some info Gary Maxwell and Gary Glassmoyer use with

their (GEOS?) data:

(21)      Sequence number of event on tape,

(22)      Sequence number of file on tape.                         &

&

IHEAD(23-24):  File creation time:                                      &

(23)      year, 4 digits,                                          &

(24)      julian day.                                              &

&

IHEAD(27-29):  info about the recorder channels:                        &

(27)      first active channel,                                    &

(28)      channel that produced the data in this file,             &

(29)      total number of channels.                                &

+ IHEAD(30)      Number of components recorded for station ID.

* IHEAD(31)      Number of data blocks in this file.  (header blocks

are not included in this count.)

* IHEAD(32)      Index of the last datum in the last data block.

IHEAD(33)      Null counter

(I think this means the number of "drop-out", or

"undefined" values found among the time series

samples.)

IHEAD(34-36):  Playback program info.  "Playback" is the process        &

in which data is read from the tape cartridges written   &

by a digital recorder onto the mainfraime (PDP/11-70     &

or VAX) disk.  Some cleanup and rearranging is           &

performed in the process.                                &

(34)      Playback program code (LT) <what shall we use for AGRAM?>&

(35)      Playback program version,                                &

(36)      Playback program subversion.                             &

&

IHEAD(37-39):  Recorder info:                                           &

(37)      Recorder type code (LT),                                 &

(38)      Recorder version,                                        &

(39)      Recorder subversion.                                     &

&

IHEAD(40-43):  Sensor info:                                             &

(40)      HV2.0: Sensor serial number.                             &

.                HV1.-: 'FB' or 'VT' to indicate whether the sensor       &

is a force-balance accelerometer or a velocity    &

transducer.                                       &

*      (41)      HV2.0 and HV1.1: Vertical orientation, integer degrees   <

from UP (this will be 90 in data files            <

representing horizontal motion).                  <

HV1.0: 'HR' or 'VR' to indicate whether the time         <

series data represents horizontal or vertical     <

motion.                                           <

*      (42)      Horizontal orientation, integer degrees clockwise

from north (This will be 0 in data files

representing vertical motion).

(43)      HV2.0: Sensor type code (LT)                             &

*** really ought to show the commonly used        &

values here.  Linda Seekins has a list ***    &

*                HV1.-: 'AC', 'VL', or 'DP' (acc, vel, or disp) or        &

'BU' for BUTTER data, 'IR' for SCALE data.        &

IHEAD(44)      HV2.0 and HV1.1:  This item is no longer used.           <

HV1.0:  Before November 1982, this was set to + or - 1   <

to indicate polarity of vertical motion.         <

Vertical motion is now described by IHEAD(41).   <

IHEAD(50)      Recorder location number.                                &

IHEAD(51)      Tape number.                                             &

IHEAD(52)      Trigger algorithm code (LT).                             &

&

IHEAD(60-63)   AGRAM pad information.  These four cells are used by

the ADDPAD and REMPAD programs that are sometimes

used before and after CORAVD.  These cells will also

eventually be used by the AVD program that will someday

replace CORAVD.  They indicate the last sample in the

leading pad and the first sample in the trailing

pad:

(60)     number of the block containing the last point in

the leading pad;

(61)     index of that last point within its block;

(62)     number of the block containing the first point in

the trailing pad;

(63)     index of that first point within its block.

IHEAD(208-209) Study information:                                       &

(208)     Study ID code (LT),                                      &

(209)     Sub-study code (LT?).                                    &

&

IHEAD(210-216) file information (for use in case the file is ever       &

moved or archived somewhere in a manner such that the    &

original file name and directory is lost)                &

(210-216) maybe: file name, hollerith ASCII characters.            &

(217-219) maybe: study name <is this the sort of info that is      &

normally conveyed by the name of the directory    &

in which the file is stored?>                     &

(220-235) maybe: current filename - extended format <what does     &

that mean?>                                       &

&

IHEAD(252)     Clock type code (LT)                                     &

(i.e., 0 = no clock; 1 = WWVB; 2= external master;       &

3 = manual; ...)                                  &

IHEAD(253)     Recording type code (LT)                                 &

(i.e., 0 = continuous; 1 = tirgger; 2= preset;           &

3 = generic calibration;                          &

4 = amplifier calibration;                        &

5 = sensor calibration; ...)                      &

IHEAD(254)     Ground motion unit code (LT)                             &

(i.e., 1 = acceleration in cm/sec/sec;                   &

2 = velocity in cm/sec;                           &

3 = displacement in cm;                           &

and maybe others for odd-ball stuff like          &

the BUTTER files.)                            &

IHEAD(255)     Spatial component number.                                &

<1,2,3 instead of the 4,5,6 and 7,8,9 shown in the       &

file names.>                                            &

*IHEAD(200-256) are used differently in (IMPORTed from text to bb)

BUTTER output files than in the DRD files and in

other AGRAM files.  For IMPORTed BUTTER files only:

*(200)      Number of digitized traces in the file.

(There are usually 3 data traces, 1 or

2 reference traces, and 1 time tick

trace.)

*(201)      Type of trace,

1-> time-tick,

2-> reference line, and

3-> data.

*(202)      Number of the first data block for the

trace.

*(203)      Number of data blocks for the trace.

*(204-?)    Repeat items (201),(202), and (203) for

each trace.

8.2  First Real Header Block

============================

The first real header block is usually the second block on the

file.  It contains 128 real (4-byte) values.

----- is the cell used by the AGRAM programs?

|

|

|  header

v location       contents

- --------       --------

* RHEAD(1)       Number of optional real header blocks that follow

this one.

* RHEAD(2)       Real value that represents "undefined" or "null".        |

On the VAX we originally used = -0.3e-38 for this     |

value, but when we learned that this value            |

can't be represented on some computers and some       |

compilers, we began using +1.7e38.                    |

The PC versions of the AGRAM programs will not        |

permit any values other than 1.7e38 in RHEAD(2),      |

except that they will change any -0.3e-38 comming     |

in from VAX BBF files to 1.7e38.                      |

The PC/AGRAM programs restrict real undefined         |

to 1.7e38 so they can inspect the value in            |

RHEAD(2) to determine whether:                        |

a) the file orignated on a VAX, in which              |

case the PC/AGRAM programs will then change        |

all the real numbers comming in from the file      |

from the VAX real-number representation (DEC)      |

to the PC representation (IEEE); and               |

b) the file is a BBF-format file or a SMC-format      |

file.                                              |

* RHEAD(5)       Sampling rate, samples/second.  If undefined, then

it's output data from BUTTER or SCALE.

. RHEAD(6)       Component sample lag (seconds).                          &

This refers to the difference between the time of

the first sample in this file and the time of the

first sample recorded by the first active channel

connected to the same recorder as recorded this

data.  Samples are taken at a different instant

by each channel.  For example, a recorder with a

.005 second sampling interval and 6 channels would

have a component sample lag of 0.0 for the first

channel, of 0.0008333 (= .005/6) for the second

channel, 0.0016667 for the third channel and so

forth.

RHEAD(10-19)   Source (EQ or explosion) information:

.      (10)      source latitude, degrees

.      (11)         "   latitude offset, minutes north of RHEAD(10)

<that "offset" is Chuck's term.  Maybe he

means to allow RHEAD(10) to contain decimal

fractions of a degree in some cases?>

.      (12)         "   longitude, degrees

.      (13)         "   longitude offset, minutes east of RHEAD(12)

(14)         "   Depth (in kilometers?)

(15)         "   Magnitude (?)

(16)         "   Moment (?) <these question-marks are Chuck's.

do they indicate that he doesn't

know what units or type to use or

do they indicate that he's not

sure he wants us to keep the

item in the file?>

(17)         "   Weight of explosives (?)

(18)         "   Origin time,

(seconds after IHEAD(10).IHEAD(11)?)

(19)         " -to-station distance? (km)

(20)         " -to-station azimuth, take-off angle info?

(21)                         "

(22)                         "                                     &

(23)                         "                                     &

&

RHEAD(24)      P-wave arrival time, seconds after FST.                  &

(FST =first sample time)                               &

RHEAD(25)      S-wave arrival time (seconds after FST)                  &

RHEAD(40-45)   Sensor position info:

(40)      Sensor latitude, degrees

(41)         "       "     offset, minutes north of RHEAD(40)

(42)         "   longitude, degrees

(43)         "       "      offset, minutes east of RHEAD(42)

(44)         "   elevation - origin <that's Chuck's "origin",

what does it mean?>, meters above

sea level.

(45)       HV2.0: Sensor elevation offset, Meters below            &

RHEAD(44).                                       &

HV1.-: Voltage input.  <Neither Chuck nor April know    &

what that  "voltage input" was supposed to       &

mean.>                                           &

RHEAD(46)      Digitizing constant.

.                This is instrument counts/volt for DRD data

(about 200.), and

+                Digitization units/cm for AGRAM data (usually 10000.).

* RHEAD(47-48)   Anti-alias filter characteristics.

(We may decide to keep only the DRD anti-alias

filter characteristics here and move the similar,

but not completely equivalent, AGRAM/HIFRIC filter

info elsewhere, perhaps to RHEAD(114) and

RHEAD(115).)

*      (47)      Anti-alias filter's corner frequency, Hz.  (in

AGRAM files, this is the freqency used with the

combined instrument correction and anti-alias

filter in HIFRIC.  Other high-cut filters may be

applied in CORAVD, but those filter characteristics

are described in RHEAD(104 - 113).)

*      (48)       HV2.0: Anti-alias filter poles (roll-off = 6dB/pole)

HV1:   same as for HV2.0 in DRD files, but in AGRAM

files this number represents the anti-alias

filter's rolloff bandwidth in Hz.

<We haven't decided where AGRAM should put

this value into V2.0 files yet, but Chuck

doesn't think he wants it here.>

RHEAD(49-51)   Sensor characteristics

+*      (49)      HV2.0: Sensor natural frequency, Hz.                       &

HV1  : As for HV2.0 in DRD files, but is the natural       &

period (NOT frequency), in seconds, in AGRAM        &

HV1 files.                                          &

+*      (50)      Sensor damping coefficient, fraction of critical           &

damping.  <Chuck's list shows % rather than         &

fraction, but I think that's just a mistake         &

he intends to fix.>                                 &

+*      (51)      Sensor sensitivity.

In DRD files, this is the coil constant in volts/unit

(unit=cm/sec/sec or cm/sec).

In AGRAM files, this is the transducer sensitivity

in cm/g.

. RHEAD(52)      Digital recorder amplifier gain, dB.

(There is no counterpart to this with AGRAM's analog

data).

RHEAD(53)      Pre-amp gain, dB.  < what's this?? should it get

included when calculating the counts-to-cm/sec/sec

conversion factor??>

RHEAD(54-57)   Analog filter info, HV2.0.  This info is stored in       &

RHEAD(104 -107) in AGRAM HV1 files.                      &

<That "analog" comes from Chuck's list.  What does       &

he mean by it?  AGRAM data?>                            &

*      (54)      HV2.0: Lo-cut filter corner frequency, Hz.               &

*      (55)      HV2.0: Lo-cut filter poles (rolloff = 6 dB/octave        &

per pole)                                               &

<so where shall AGRAM put the rolloff bandwith when      &

something other than a Butterworth filter is used?      &

just leave it in RHEAD(106)? >                          &

*      (56)      HV2.0: Hi-cut filter corner frequency, Hz.               &

*      (57)      HV2.0: Hi-cut filter poles.                              &

<same problem.  See RHEAD(107)>                          &

RHEAD(60-61)   Clock correction info.  <Does this mean the timing

info in the recorder gets recalibrated at intervals

while it is running, or is a clock correction

something that is learned after data is recorded?>

.      (60)      Clock correction, seconds.  *** BEWARE:  Chuck warns

that the clock correction values found in the

headers are often (usually?) unreliable. ***

.      (61)      Time since last clock correction (s?)                    &

&

RHEAD(62)      I don't know what is in here but it was filled in        &

in some of Mehmet's Whittier Narrows EQ records.       &

&

RHEAD(63-72)   HV2.0: Gary Maxwell's trigger info.                      &

RHEAD(70-71)   HV1.-: P and S wave picks

RHEAD(73-89)   Appear to be empty.

RHEAD(90-97)   min/max, begin/end info.

This info has always been kept here (cells #90-97) in    &

the AGRAM files.  Are the DRD folks intending to      &

fill this space with the same sort of info in the     &

future?                                               &

The times given in this section are in seconds

relative to the first recorded sample, as though

that first sample occurred at time = 0.0.  None

of the timing info in RHEAD(60-61) or IHEAD(10-16)

is included in these "times".

The ordinate values given in this section, for the

AGRAM files at least, are in cm/sec/sec, cm/sec,

or sec.  <I'm not sure any of the DRD folks

will want to fill in this section, but if so,

will the values here be in metric units or in

instrument counts?>

<Chuck didn't include RHEAD(90-93) in his list, but

I wish he would so nobody will use those cells

for something else in the future.>

*      (90)      Time of first sample in this file relative to the

time of the first sample recorded.  This is

usually = 0.0 or "undefined", but will have some

other value in those cases where an AGRAM program

has removed or added points to the beginning of a

time series.  This will occur primarily in files

produced by the AVD program (or its temporary

predecessor ADDPAD) that include the pre-filter

pad in the output file: it will be a negative

number representing the length of the leading pad.

*      (91)      Value of the first data point.

*      (92)      Time of the last sample,

*      (93)      Value        "         ,

*      (94)      Time of the maximum value.

*      (95)      Value    "

*      (96)      Time of the minimum value.

*      (97)      Value    "

* RHEAD(98)      HV2.0: Trace mean value.  This value should be very      &

close to zero for all but SCALE output files.     &

*** IMPORTANT:  This value must be subtracted     &

from all the data samples by any program that     &

processes SCALE output files, for SCALE does      &

NOT do so itself.  The data in files produced     &

by programs after SCALE in the AGRAM process      &

is OK, for those programs have subtracted out     &

the scale offset before any other processing.     &

*      (98)      HV1.-: Mean value of the SCALE data.  This value is      &

subtracted from SCALE data by any AGRAM program   &

that processes SCALE output files (HIFRIC,        &

TSPLOT, EXPORT) but they leave the SCALE offset   &

value in RHEAD(98) of their output files.         &

The following stuff is for AGRAM only, Chuck's list doesn't show

anything after RHEAD(98).  I'd like Chuck to show these cells as

reserved for AGRAM stuff, like he reserved RHEAD(63-72) for Max's

trigger stuff.  Or maybe we could declare RHEAD(99-128) to be a

"scratch" area as HV1 did, so each type of data file could have its

own conventions there.  We would need to restrain ourselves from

storing anything important in there.  All this AGRAM trash in 99-128

is used only for reference and/or creating plot labels.

* RHEAD(99-102)  Info filled in by the AGRAM/CORAVD program:

*      (99)      Beginning time for the least squares fit baseline

correction.

*      (100)     Ending time for the least squares fit baseline

correction.

*      (101)     Slope of the linear baseline correction.

*      (102)     Intercept of the linear baseline correction.

* RHEAD(103)    AGRAM source-of-data indicator.  Note that the TSPLOT

program uses source of data =

mod( abs(RHEAD(103)), 20.0)

when setting up the plot labels via the "SEB" option.

=  1.0 if from SCALE;

-1.1 if from ST6PLT;

2.0 if from HIFRIC, with interpolation only;

2.1 if from HIFRIC, with time-domain instrument

correction and high-cut filter;

2.2 if from HIFRIC, with frequency-domain

instrument correction and high-cut filter;

3.0 if from CORAVD, without long-period filter or

linear baseline correction to velocity;

3.1 if from CORAVD, without filter, with linear

baseline correction;

3.2 if from CORAVD (or SLOWAVD) with filter and

linear baseline correction.

4.x if from BAP.  x=> outout from BAP's step # =x.

5.x if from ROTATE.  x= whathever number was in

RHEAD(103) before the rotating, or =0 if the

two files had different values in RHEAD(103).

(and maybe someday:

0.0 => this is a BUTTER output file;

20.+=>  "        COMBINE  "        ;

40.+=>  "        ROTATE   "        ;

60.+=>  "        IMPORT   "        .

* RHEAD(104-113)  Filter info filled in by the AGRAM/CORAVD program:

*      (104)      HV1.-: Low-cut filter roll-off or corner, cps.

HV2.0: Shall we move or duplicate this info in          &

RHEAD(54)?                                       &

*      (105)      HV1.-: High-cut filter roll-off or corner, cps.

HV2.0: Shall we move or duplicate this info in          &

RHEAD(55)?                                       &

*      (106)      HV1.-: Low-cut filter rolloff band width, cps, or

integer roll-off parameter (depending on the

type of filter used.

Note: the roll-off parameter, M, given as

input to CORAVD Butterworth filters

is 1/2 the "order", N, of the

Butterworth filter.  And there

are 2 poles/order.  So number of poles

= 2N = 4M.  Thus, the rolloff parameter

used here to characterize a filter's

roll-off is 1/4 the number (= # of

poles) that the DRD folks would use

to characterize the roll-off in an

equivalent filter.  See RHEAD(48),

RHEAD(55), and RHEAD(57).

HV2.0: Chuck wants number of poles in  RHEAD(55),       &

but where will I put transition info if it       &

isn't a Butterworth filter??  Just keep it       &

right here, as always, in RHEAD(106)?            &

*      (107)      HV1.-: High-cut filter rolloff band with, cps,

or integer roll-off parameter.

HV2.0: <see RHEAD(106>                                  &

*      (108)    Low-cut filter type indicator,

*      (109)    High-cut filter type indicator,

=1.,2.,3.  or 4.  to indicate unidirectional

Butterworth filter, bidirectional Butterworth

filter, Ormsby filter, or FFT filter.

*      (110)    Low-cut filter padding identifier

(1.=> zero fill, 2.=> cosine fill, ...  others, too)

*      (111)    High-cut filter padding identifier.

*      (112)    Low-cut Ormsby filter option,

*      (113)    Reserved for high-cut Ormsby filter option, just in

case high-cut Ormsbys are ever added to CORAVD.

?      (114)    Shall we put the HFRIC high-cut filter frequency

here that HV1 HFRIC now sets into RHEAD(47)?

?      (115)    Shall we put the HFRIC high-cut filter transition

information here that HV1 HFRIC now sets into

RHEAD(48)?

?      (114)    Shall we put a type of filter indicator here for

the HFRIC high-cut filter, similar to the

CORAVD filter indicators in RHEAD(108) and (109)?

Really isn't necessary, since the present version

of HIFRIC always uses the same filter.

9.0  BBFILE and TSPLOT: programs that display the contents of bb files

======================================================================

The BBFILE program can be used to dump all the header blocks and

selected data blocks in a data file.  BBFILE is an AGRAM program, so

you must define all the AGRAM symbols in order to use it.  To do so,

just:

$ @PUB1:[AGRAM]AGRAM.DEF

Then, to use BBFILE, try:

$ BBFILE <data file name> !dump will appear on the terminal

display

$ BBFILE TMP.TMP= <datafile name>,1,3-5  !the dump will go

into the file named TMP.TMP and

data blocks #1, 3,4,and 5 will be

dumped in addition to the header

blocks.

or  $ HELP BBFILE

Another AGRAM program that can be used to show the content of a

blocked binary data file is TSPLOT.  To get a plot of the time series

in a blocked binary data file, type:

$ TSPLOT <filename>

For more information about TSPLOT, type:

$ HELP TSPLOT

BBFILE and TSPLOT, like most the AGRAM programs, can process DRD

files in addition to the AGRAM files.